ELO  XPE PACKAGE

Scope:

This document describes the ELO_XPE_UX86 components. This includes both  USB and Serial components.

Assumptions:

The user has the system with Windows XP Embedded Development Environment with Service Pack2. User has the thorough  knowledge of integrating the XPE components into the target environment.  User has the knowledge of the Elotouch Technology.

Description:

ELO_XPE_UX86.SLD file contains  the Elo Windows XPE driver package. It contains all the files needed to embed the Elo Touchscreen drivers and applications into and XPE OS. All Elo components are grouped under  two categories, "Human Interface Devices" and   "Mice and Other Pointing devices".  The binary files are built and tested for XP Professional OS. For the functionality of the driver please refer to the XPU Driver User’s Manual. This is not included with this component. You need to refer to the XP Professional driver for the users manual.

Files and Dependencies:

The files are listed in the EloDrvi386Rep sub-directory. The dependencies for these components are listed as below.  Each of the dependency is described. Some of the dependencies are optional, for example beep driver, which is needed only for beep functionality.

| Beep Driver | Optional:Required for Beep Otherwise Beep won't work |
| --- | --- |
| Control Panel Command Line Support | Optional:Required for Launching Control Panel if not No control panel |
| Device: HIDClass | Required for the EloUSB |
| Device: Mouse | Required as the touch driver INF file includes it and uses the mouse functionality |
| HID-compliant mouse | Required as the touch driver INF file includes it and uses the mouse functionality |
| Mouse Class Driver | Required as the touch driver INF file includes it and uses the mouse functionality |
| Primitive: Cdfview | Required for Windows |
| Primitive: Hid.dll | Required for the EloUSB |
| Primitive: Hidclass.sys | Required for the EloUSB |
| Primitive: Mpr | Required for Windows |
| Primitive: Regsvr32 | Required for registering new DLLs |
| Primitive: Rundll32 | Required for Launching Control Panel and running other DLLs |
| Primitive: Shlwapi | Required for Launching for some basic Windows Functionality |
| Primitive: W32topl | Required as timer API depends on it |
| Service Command Line Tool | Required for Launching the service by the user |
| Shell Notepad Registry Data | Required for using Notepad |
| Tray Icon Add/Remove Support | Optional:Required for Tooltray Support to be available. |
| USB Human Interface Device | Required: Function driver for the EloUSB |
| WMI Core | Optional:Required for Right Click On Hold. |
| WMI Extensions to WDM Property Page Provider | Optional:Required for Right Click On Hold. |
| WMI Win32 Provider | Optional:Required for Right Click On Hold. |
| Explorer Shell | Optional: Required for Launching the EloDKMon etc from EloSrvce |
| Internet Explorer | Optional:Required for reading the User's manual |

Instructions to Add the  USB components:

- Open the Component Database Manager and Import ELO_XPE_UX86.SLD

- Close the Component Database Manager

- Open  Target designer  and Open  your configuration file

- Add  the component "Elo TouchSystems USB Touchmonitor Interface " under  the Human Interface Devices category, to your configuration.

- Add the component “Elo TouchSystems MouHid Filter”  from the Mice and Other Pointing devices category.

- Resolve the dependencies and build the image.

Instructions to Add the  Serial components:

- Open the Component Database Manager and Import ELO_XPE_UX86.SLD

- Close the Component Database Manager

- Open  Target designer  and Open  your configuration file

- Add the component  “Elo Serial Touchmonitor Interface” from Mice and Other Pointing devices category, to your configuration.

- Resolve the dependencies and build the image.

Precalibration:

A text file  EloCompOptions.Txt file is included in the component for enabling the some options. To enable Precalibration, we need to edit this  before we launch the image and after the image is built. Go to your image directory and find this file in Pragram files\EloTouchsystems directory. Open the file with Notepad or any Text only editor.

Change the number in the following line to 1, if you want to enable the Precalibration.

PreCal,0

If for some reason you don't see the effect, you can verify the registry value "WriteNVRam" at the key "HKEY_CURRENT_USER\Software\Elo TouchSystems". That should be exactly what you set in the above text file.

Multiple Serial Ports:

The Elo Serial Touchmonitor Interface component by default adds 1  serial port at installation. Add support for multiple ports by editing EloCompOptions.Txt  file.The following  two lines are to be modified. To change the number of ports, change the NumPorts value to the desired value.If you need to change  port,change the FriendlyName value.  Generally friendly name is like Communications Port (COMx), where x is the number of the port. If this string doesn't match with the one in the system, the serial port doesn't work. So it is very important that you have exactly matching string for the friendly name.

NumPorts,1

FriendlyName,Communications Port (COM1)

This procedure effectively changes

- Adds  the registry key "HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\EloTouchscreen\LegacyPorts\COM1"

- Add the string value name to the above key :  FriendlyName and Communications Port (COM1).

- Modify "HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\EloTouchscreen\InstalledSerialDevCnt"  to 1

You need to add as many lines of friendlynames as many ports. For example for two ports on COM1 and COM2 you need to have these lines in the file.

NumPorts,2

FriendlyName,Communications Port (COM1)

FriendlyName,Communications Port (COM2)

Font Problems:

You need to have the MS Sans Serif, Tahoma  and Arial fonts for our languages component to function properly. Some languages don’t have these fonts.